- Autofluorescence Imaging

Autofluorescence imaging (AFI) is based on the detection of natural tissue fluorescence emitted by endogenous molecules (fluorophores) such as collagen, flavins, and porphyrins. After excitation by a short-wavelength lightsource, these fluorophores emit light of longer wavelengths (fluorescence). The overall fluorescence emission differs among various tissue types due to correspondingdifferences in fluorophore concentration, metabolicstate, and/or spatial distribution

For example, illuminating the retina with blue light causes certain cellular components to “glow” without injecting any dye:

- Bright-field microscopy

The simplest of all the optical microscopy illumination techniques. Sample illumination is transmitted (i.e., illuminated from below and observed from above) white light, and contrast in the sample is caused by attenuation of the transmitted light in dense areas of the sample. Bright-field microscopy is the simplest of a range of techniques used for illumination of samples in light microscopes, and its simplicity makes it a popular technique. The typical appearance of a bright-field microscopy image is a dark sample on a bright background, hence the name.

- Epifluorescence microscopy

The majority of fluorescence microscopes, especially those used in the life sciences, are of the epifluorescence design shown in the diagram. Light of the excitation wavelength illuminates the specimen through the objective lens. The fluorescence emitted by the specimen is focused to the detector by the same objective that is used for the excitation which for greater resolution will need objective lens with higher numerical aperture. Since most of the excitation light is transmitted through the specimen, only reflected excitatory light reaches the objective together with the emitted light and the epifluorescence method therefore gives a high signal-to-noise ratio.

More info: https://en.wikipedia.org/wiki/Fluorescence_microscope

- Fluorescence Reconstruction Microscopy (FRM)

Process that takes in a transmitted light image of a biological sample and outputs a series if reconstructed fluorescence images that predict what the sample would look like had it been labelled with a given series of dyes or fluorescently tagged proteins.

FRM works by first training a CNN (e.g., U-Net) to relate a large set of transmitted light data to corresponding real fluorescence images (ground truth) for given markers.